ENTERPRISE HARDWARE & DEVICE INVENTORY SPECIFICATION

NexaTech Solutions Pvt. Ltd. — 3-Floor Corporate Headquarters
Author: Member 1 (Network Design Engineer) | Project: CNDC Enterprise Network

1. Equipment Selection Executive Summary

This document details the complete hardware bill of materials, device placements, and technical port specifications for NexaTech Solutions Pvt. Ltd. The enterprise infrastructure is engineered based on the Cisco Hierarchical 3-Tier Campus Model (collapsed Core-Distribution design) supporting 140 corporate staff across 6 departments, an isolated guest network, and centralized servers. All components have been selected to ensure wire-speed inter-VLAN routing, perimeter defense, robust wireless mobility, and 100% compatibility with Cisco Packet Tracer simulation environments.

2. Comprehensive Hardware Device Inventory

| Device Name | Model / Spec | Network Role | Port Capacity | Qty | Physical Location | VLAN / Function |
| --- | --- | --- | --- | --- | --- | --- |
| R1-Edge-2911 | Cisco 2911 ISR | WAN Edge Router | 3x GE (10/100/1000), 4x EHWIC | 1 | Floor 2 Server Room (MDF Rack 1) | WAN / Gateway NAT to ISP |
| FW-Perimeter-5506 | Cisco ASA 5506-X | Perimeter Firewall | 8x GE (10/100/1000), 1x Console | 1 | Floor 2 Server Room (MDF Rack 1) | DMZ & Inside/Outside Perimeter Security |
| SW-Core-3560 | Cisco Catalyst 3560-24PS | Core Multilayer Switch | 24x 10/100 PoE, 2x SFP Gigabit | 1 | Floor 2 Server Room (MDF Rack 1) | Central Layer 3 SVIs & Inter-VLAN Routing |
| SW-ASW1-Floor1 | Cisco Catalyst 2960-24TT | Floor 1 Access Switch | 24x 10/100 FastEth, 2x GE Uplinks | 1 | 1st Floor IDF-2 Rack | Access Layer: HR (VLAN 10) & Finance (VLAN 20) |
| SW-ASW2-Ground | Cisco Catalyst 2960-24TT | Ground Floor Access Switch | 24x 10/100 FastEth, 2x GE Uplinks | 1 | Ground Floor IDF-1 Rack | Access Layer: Support (VLAN 50), Sales (VLAN 40) |
| SW-ASW3-Floor2 | Cisco Catalyst 2960-24TT | Floor 2 Access Switch | 24x 10/100 FastEth, 2x GE Uplinks | 1 | 2nd Floor IDF-3 Rack | Access Layer: IT Dept (VLAN 30) & Mgmt (VLAN 60) |
| SRV-DHCP-DNS | Server-PT (Enterprise Server) | Infrastructure Server 1 | 1x FastEth / Gigabit NIC | 1 | Floor 2 Server Room (MDF Rack 2) | VLAN 100: Centralized DHCP Pools & DNS Resolver |
| SRV-WEB-FILE | Server-PT (Enterprise Server) | Application Server 2 | 1x FastEth / Gigabit NIC | 1 | Floor 2 Server Room (MDF Rack 2) | VLAN 100: Intranet Web Server & FTP File Store |
| WAP-GF-Guest | Cisco AccessPoint-PT | Guest Wi-Fi AP | 1x FastEthernet, 802.11b/g/n | 1 | Ground Floor Reception Ceiling | VLAN 70: Isolated Visitor Wireless Access |
| WAP-1F-Corp | Cisco AccessPoint-PT | Floor 1 Corporate AP | 1x FastEthernet, 802.11b/g/n | 1 | 1st Floor Center Ceiling | Corporate Wireless Mobility (Sales/HR) |
| WAP-2F-Exec | Cisco AccessPoint-PT | Floor 2 Executive AP | 1x FastEthernet, 802.11b/g/n | 1 | 2nd Floor Boardroom Ceiling | Executive & IT Wireless Mobility |
| Representative PCs | PC-PT / Desktop Workstation | Department Endpoints | 1x FastEthernet NIC | 14 | Distributed across all 3 floors | Simulate 140 staff workstations (2-3 per Dept) |
| Representative Laptops | Laptop-PT (Mobile Client) | Executive/IT Laptops | 1x FastEthernet, 1x Wi-Fi NIC | 4 | Executive Suite, IT Lab, Guest | Simulate mobile workstations and guest clients |

3. Device Justification & Sizing Rationale

• Cisco 2911 Integrated Services Router (ISR): Selected for its robust modular architecture, integrated hardware-accelerated encryption, 3 onboard Gigabit Ethernet interfaces for WAN termination, and full support for advanced BGP/OSPF dynamic routing and NAT overloading. Acts as the hardened edge boundary between the ISP and corporate intranet.

• Cisco ASA 5506-X Next-Generation Firewall: Deploys stateful packet inspection, deep packet inspection, and adaptive access control between the external untrusted WAN and internal trusted campus LAN. Configured with Security Level 0 (Outside) and Security Level 100 (Inside) to guarantee network segmentation and enforce corporate ACL policies.

• Cisco Catalyst 3560-24PS Multilayer Switch: Functions as the core distribution backbone. Performing wire-speed Inter-VLAN routing directly on hardware SVIs eliminates the 'Router-on-a-Stick' throughput bottleneck. Equipped with Power-over-Ethernet (PoE) capabilities and 802.1Q trunking to feed access switches and direct gigabit connections to the server farm.

• Cisco Catalyst 2960-24TT Access Switches: Delivers line-rate FastEthernet connectivity to client desktop workstations with dual GigabitEthernet uplink interfaces dedicated for 802.1Q trunking back to the core switch. Supports 802.1p Quality of Service, port security, and rapid Spanning Tree Protocol (RSTP) to eliminate loop topologies.

• Centralized Enterprise Dedicated Servers (Server-PT): Server 1 consolidates core network identity services (centralized multi-scope DHCP and local DNS forward/reverse resolution for intranet.nexatech.com). Server 2 hosts internal web portals (HTTP/HTTPS) and centralized department FTP document sharing. Grouped inside isolated VLAN 100 to enforce strict ACL security.

4. Structured Cabling & Physical Infrastructure Standards

| Infrastructure Tier | Media Type | Standard / Category | Max Distance | Connector Type | Application / Location |
| --- | --- | --- | --- | --- | --- |
| Horizontal Workstation Cabling | Copper UTP 4-Pair | Category 6 (ANSI/TIA-568-C.2) | 90 meters + 10m patch | RJ-45 (8P8C) | IDF Patch Panels to Desk Faceplates on all 3 Floors |
| Vertical Riser Backbone | Multi-Mode Fiber / Cat6A | OM3 50/125 µm / 1000BASE-SX | 300 meters | LC Duplex / RJ-45 | Vertical Riser Shaft connecting MDF Core Switch to IDFs |
| Server Farm Interconnects | High-Grade Cat6 Patch | Category 6 Snagless | 5 meters | RJ-45 (Shielded) | Core Switch Gi/Fa ports to Server 1 & Server 2 NICs |
| WAN Transit Links | Single-Mode Fiber / Cat6 | 1000BASE-T / 1000BASE-LX | Up to 5 km | SC/LC / RJ-45 | ISP Optical Termination to Cisco 2911 ISR Gi0/0 |
| Rack Equipment Patching | Cat6 Molded Patch Leads | Category 6 Stranded | 1 - 3 meters | RJ-45 (TIA-568B) | Within MDF Core Rack and IDFs for Switch-to-Panel links |

5. Rack Elevation & Equipment Allocation

To ensure thermal efficiency, maintainability, and clean structured cable routing, the following physical rack allocations are established:
• MDF Main Rack (42U Floor-Standing, Server Room, 2nd Floor): Houses WAN fiber patch box (40U-41U), Cisco 2911 Router (38U), Cisco ASA 5506-X (36U), Horizontal Cable Organizer (35U), Cisco Catalyst 3560 Core Switch (34U), 24-Port Cat6 Patch Panels (31U-33U), Server 1 (28U-29U), Server 2 (26U-27U), and 3000VA Online Double-Conversion UPS (1U-4U).
• IDF-1 Wall-Mount Cabinet (12U, Ground Floor Wiring Closet): Houses 24-Port Cat6 Patch Panel (10U), Cable Manager (9U), Cisco 2960 Access Switch 2 (8U), and 1000VA UPS (1U-2U).
• IDF-2 Wall-Mount Cabinet (12U, 1st Floor Wiring Closet): Houses 48-Port Cat6 Patch Panel (10U-11U), Cable Manager (9U), Cisco 2960 Access Switch 1 (8U), and 1000VA UPS (1U-2U).
• IDF-3 Wall-Mount Cabinet (12U, 2nd Floor Wiring Closet): Houses 24-Port Cat6 Patch Panel (10U), Cable Manager (9U), Cisco 2960 Access Switch 3 (8U), and 1000VA UPS (1U-2U).