ENTERPRISE NETWORK REQUIREMENTS SPECIFICATION

Project: NexaTech Solutions Corporate Infrastructure | Phase 1 Group Baseline
Author: Member 1 (Network Design Engineer) | Contributing: Members 1, 2 & 3

1. Company Scenario & Business Context

NexaTech Solutions Pvt. Ltd. is an emerging medium-sized information technology and software development company experiencing rapid corporate growth. The organization has recently acquired and occupied a modern three-story commercial office building to house its primary engineering, customer delivery, commercial, and executive leadership teams. The organization currently employs 140 professionals and regularly accommodates external clients, vendor partners, and remote technical staff. To support mission-critical day-to-day operations, NexaTech requires an enterprise-class, resilient, scalable, and highly secure local area network (LAN) infrastructure.

2. Departmental Structure & User Population

| Department Name | Target VLAN | Staff Count | Floor Location | Access Switch Association | Operational Function |
| --- | --- | --- | --- | --- | --- |
| Human Resources (HR) | VLAN 10 | 15 | 1st Floor (West Wing) | SW-ASW1-Floor1 | Personnel management, recruitment, private employee records |
| Finance & Accounting | VLAN 20 | 20 | 1st Floor (East Wing) | SW-ASW1-Floor1 | ERP access, corporate payroll, audit compliance, financial reporting |
| Information Technology (IT) | VLAN 30 | 25 | 2nd Floor (South Wing) | SW-ASW3-Floor2 | Network engineering, cloud DevOps, systems administration, helpdesk |
| Sales & Marketing | VLAN 40 | 30 | Ground / 1st Floor | SW-ASW2-Ground | Client engagement, CRM database access, marketing media streaming |
| Customer Support Center | VLAN 50 | 40 | Ground Floor (Main Hall) | SW-ASW2-Ground | 24/7 client ticketing, high-volume VoIP calling, CRM queries |
| Executive Management | VLAN 60 | 10 | 2nd Floor (Executive Suite) | SW-ASW3-Floor2 | C-suite strategic leadership, board conferences, confidential docs |
| Guest / Visitor Network | VLAN 70 | Dynamic (30+) | Ground Floor Lobby & Lounges | SW-ASW2-Ground (via WAP) | Public Wi-Fi access for visitors, strictly isolated from LAN |
| Enterprise Server Farm | VLAN 100 | Dedicated Servers | 2nd Floor MDF Server Room | SW-Core-3560 (Direct GE) | Centralized corporate DHCP, DNS, Intranet Web & File sharing |

3. Functional Network Requirements

• FR-01: Logical Departmental Segmentation: Each department must reside within its own dedicated Virtual Local Area Network (VLAN) to contain broadcast traffic, enforce security boundaries, and provide organized address management.

• FR-02: High-Performance Inter-VLAN Routing: Authorized inter-departmental communication and traffic destined for corporate servers must be routed at wire-speed using Switch Virtual Interfaces (SVIs) on the Layer 3 Core Switch without relying on slow external router hops.

• FR-03: Centralized Network Services Integration: All internal endpoints must automatically obtain dynamic IP configurations (IP address, subnet mask, default gateway, DNS server) via a dedicated enterprise DHCP server. Domain name resolution for intranet resources must be resolved through a private DNS server (e.g., intranet.nexatech.com).

• FR-04: Secure Guest Wireless Isolation: Visitors in the reception area must be granted Internet connectivity through dedicated Wi-Fi access points. Guest traffic must be strictly quarantined from accessing internal department VLANs, management consoles, or corporate file servers.

• FR-05: Perimeter Defense & Controlled WAN Access: The corporate network boundary must be protected by an enterprise firewall (Cisco ASA 5506-X) performing stateful packet filtering and Access Control Lists (ACLs) to block unauthorized inbound connections while enabling secure PAT/NAT for outbound Internet browsing.

• FR-06: Shared Intranet & File Collaboration: A dedicated server within VLAN 100 must host the internal HTTP company intranet portal and an FTP service for departmental file transfers, accessible only by authenticated staff.

4. Non-Functional Engineering Requirements

| Category | Requirement ID | Metric / Specification | Engineering Implementation Strategy |
| --- | --- | --- | --- |
| Scalability | NFR-01 | 200% Growth Capacity | Subnet masks engineered with Class C (/24) boundaries allowing expansion up to 254 hosts per department. |
| Performance | NFR-02 | Gigabit Backbone / 100M Access | Gigabit 802.1Q uplinks between Access switches and Core switch; 100 Mbps dedicated full-duplex to workstations. |
| Availability | NFR-03 | 99.9% Uptime for Core | MDF Server Room equipped with dual online UPS units, redundant trunk paths, and Rapid Spanning Tree Protocol (RSTP). |
| Security | NFR-04 | Zero Trust Perimeter & Layer 3 ACLs | Strict ingress/egress ACLs filtering inter-VLAN flows, disabling unused switch ports, and isolated guest network. |
| Manageability | NFR-05 | Standardized Hostnames & Syslog | Standardized naming convention (e.g., SW-ASW1-Floor1), synchronized NTP, SSH v2 management access. |

5. Project Phase & Responsibility Hand-off Matrix

| Phase & Member | Assigned Focus Area | Key Input Required | Final Deliverables Handed Off |
| --- | --- | --- | --- |
| Phase 1 & 2: Member 1 (Lead) | Network Design & Infrastructure | Company scenario, user counts, floor layout | Device inventory, physical/logical topology diagrams, Base_Topology_v1.pkt, port connection matrix |
| Phase 3 & 4: Member 2 | IP Addressing, VLANs & Routing | Base_Topology_v1.pkt from Member 1 | Subnet calculations (10.10.0.0/16), VLAN creation (10-100), 802.1Q trunks, Core Switch SVIs, Network_Config_v2.pkt |
| Phase 4 & 5: Member 3 | Enterprise Services, Security & ACLs | Network_Config_v2.pkt from Member 2 | DHCP scopes, DNS records, Web/FTP servers, Guest/Finance ACLs, end-to-end ping testing, Services_Security_v3.pkt |
| Phase 6 & 7: All Members | Final Report Integration & Viva | Individual report sections & test logs | Consolidated Technical Project Report, Packet Tracer demo file, Viva presentation deck |